Term 3 Robotics Challenge - Updated Week 1 Planning Pack

Checklist | Hardware List | Mounting Points | Revised Task Allocation | Week 1-3 Roadmap

This updated version fixes the main issues in the previous task split: Person 1 and Person 2 now have a clearer CAD/mechanism interface, Person 3 and Person 4 are explicitly paired for hardware testing, Week 1 goals are more realistic, and the plan is tightened because trial runs begin in Week 3.

# 1. Design Assumptions

Robot concept: two-wheel differential-drive car with one rear caster wheel or ball caster, similar in complexity to our Term 2 robot.

Chassis: two-layer laser-cut acrylic plate structure with standoffs/spacers and simple 3D-printed brackets.

Target size: approximately 190 mm x 170 mm x 130-140 mm, safely below the 20 x 20 x 15 cm limit.

Main objective for Week 1: not a complete robot, but a manufacturable first prototype design plus key module tests.

Main objective before Week 3 trial run: basic integrated task flow, even if not fully optimised.

# 2. Robot Design Checklist

| Requirement | Proposed solution | Priority / status |
| --- | --- | --- |
| Maximum size: 20 x 20 x 15 cm | Target design: 190 x 170 x 130-140 mm | Must confirm in CAD |
| Hardware kill switch | Top-mounted red button, easy to press | Required |
| Software kill switch over WiFi | Implemented in Arduino state machine / communication logic | Required |
| Line detection | Front-bottom reflectance sensor array on adjustable bracket | Required |
| Wall/distance detection | Front ToF distance sensor; optional side ToF if time allows | Required |
| Battery power | Battery mounted low and central on lower deck | Required |
| No breadboard in finals | Week 1 testing can use jumper wires/breadboard; final must move to perfboard/soldered board/PCB | Important clarification |
| Tidy wiring | Cable routing slots, zip-tie holes, fixed electronics modules | Required |
| Planter / hopper up to 5 seeds | Low-profile angled seed hopper, 14-16 mm internal width/diameter | Required |
| Seed release | SG90 servo gate plus single-seed pocket to reduce double-release risk | Required |
| Front revive button | Wide front bumper plate with push button/micro switch behind it | Required |
| Top LED red/green | Red by default, green when revive bumper/button is pressed | Required |

# 3. Hardware List

## 3.1 Provided / expected core hardware

| Part | Purpose | Mounting location |
| --- | --- | --- |
| Arduino GIGA R1 WiFi | Main controller, WiFi communication, sensor reading, servo control | Upper acrylic deck |
| Motoron M3S550 motor shield | Controls left and right drive motors | Upper acrylic deck, close to Arduino |
| Reflectance sensor array | Line following | Front-bottom adjustable bracket |
| RFID sensor / reader | Reads RFID tags under planting holes | Under chassis, close to seed outlet |

## 3.2 Additional hardware likely needed

| Part | Purpose | Notes |
| --- | --- | --- |
| 2 DC gear motors | Left/right differential drive | Must be compatible with Motoron and chosen battery voltage |
| 2 drive wheels | Main movement | Approx. 60-70 mm diameter preferred |
| 1 rear caster wheel / ball caster | Rear support | Keeps drivetrain simple and compact |
| Battery pack | Final power source | Mount low and central for stability |
| Front ToF distance sensor | Wall, obstacle, door detection | Front-facing priority |
| Optional side ToF sensor | Wall following | Add only if time allows |
| SG90 servo | Seed gate open/close | For one-seed release mechanism |
| Push button / micro switch | Revive trigger | Behind wide front bumper |
| Red/green LED or RGB LED | Revive status indicator | Top-mounted and visible |
| Hardware kill switch | Manual emergency stop | Top-mounted, easy to press |
| Acrylic sheets | Two-layer chassis | Laser cut in lab |
| Standoffs / spacers | Separate upper and lower decks | Nylon or metal; nylon is non-conductive |
| Screws / nuts / washers | Mounting all modules | M3 hardware recommended |
| Zip ties / cable clips | Cable management | Important for inspection |
| Perfboard / soldered board | Final wiring | Finals should not use breadboard |

# 4. Mounting Points

## 4.1 Lower acrylic deck

| Component | Mounting point needed | Notes |
| --- | --- | --- |
| Left motor | Motor bracket holes | Align axle with left wheel |
| Right motor | Motor bracket holes | Align axle with right wheel |
| Rear caster / ball caster | 3-4 screw holes | At rear centre |
| Battery | Strap holes / bracket holes | Low and central |
| RFID reader | 2-4 screw holes or small bracket | Under chassis, close to seed outlet |
| Reflectance sensor array | Adjustable-height front bracket holes | Approx. 3-8 mm from ground |
| Seed outlet | Circular outlet hole | Close to RFID reader |
| Cable routing | Small slots / zip-tie holes | Avoid hanging wires |

## 4.2 Upper acrylic deck

| Component | Mounting point needed | Notes |
| --- | --- | --- |
| Arduino GIGA R1 WiFi | 4 mounting holes | Keep USB access clear |
| Motoron M3S550 | 4 mounting holes | Close to Arduino and motor wires |
| Hardware kill switch | Large circular hole | Top, easy to press |
| Red/green LED | Small LED hole / bracket | Top visible position |
| Seed hopper support | Bracket mounting holes | Rear or rear-side, low-profile |
| SG90 servo bracket | Servo screw holes | Close to hopper outlet |
| Cable management | Zip-tie holes | Keep wiring fixed |

## 4.3 Front section

| Component | Mounting point needed | Notes |
| --- | --- | --- |
| Revive bumper | Two side brackets | Wide enough for another robot to press |
| Push button / micro switch | Behind bumper | Must trigger reliably |
| Front ToF sensor | Small vertical bracket | Above line sensor |
| Reflectance sensor array | Low adjustable bracket | Must not block bumper |

## 4.4 Rear / rear-side section

| Component | Mounting point needed | Notes |
| --- | --- | --- |
| Angled seed hopper | 3D-printed support bracket | Low-profile to stay under 15 cm |
| SG90 servo gate | Servo bracket | Needs reliable open/close positions |
| Single-seed pocket | Integrated with hopper/outlet | Prevents two seeds dropping at once |

# 5. Revised Task Allocation for 4 People

This allocation is mainly for Week 1. The same roles can continue later, but the tasks should be updated after the first prototype and module tests. The most important correction is that CAD/mechanism and electronics/software are not fully independent: they must work through clear interfaces.

## Person 1: Main CAD / Chassis Integration

Design the main two-layer acrylic chassis and overall component layout.

Place motors, wheels, rear caster, battery, Arduino, Motoron, RFID reader, reflectance sensor, ToF sensor, hopper, servo, kill switch, and LED.

Integrate Person 2's hopper / servo gate / revive bumper geometry into the full chassis CAD.

Add mounting holes, standoff holes, zip-tie holes, and cable routing holes.

Keep the robot under 20 x 20 x 15 cm.

Export prototype DXF/STL files for laser cutting and 3D printing.

Deliverables: chassis CAD, upper/lower deck DXF files, mounting hole layout, size check drawing, first prototype export files.

## Person 2: Planting + Reviving Mechanism

Design the angled hopper for up to 5 seeds.

Design the SG90 servo gate and single-seed pocket.

Design the bottom seed outlet.

Design the front revive bumper and button/micro switch mounting.

Provide exact dimensions and mounting hole requirements to Person 1.

Support quick manual testing of seed flow and bumper/button triggering.

Deliverables: hopper CAD, servo gate CAD, revive bumper CAD, seed outlet design, mechanism test notes, mounting interface requirements for Person 1.

## Person 3: Electronics / Wiring

Test Arduino GIGA upload.

Wire Arduino, Motoron, motors, reflectance sensor array, RFID reader, ToF sensor, servo, LED, revive button, and kill switch.

Create pin assignment table and wiring diagram.

Plan battery power route and hardware kill switch wiring.

Plan tidy cable management and the final perfboard/soldered board solution.

Work directly with Person 4 during all module tests.

Deliverables: pin assignment table, wiring diagram, power plan, initial hardware test setup, cable management plan.

## Person 4: Software / Test Logging

Write Arduino upload/blink test.

Write motor test code.

Write reflectance sensor reading/calibration code.

Write RFID reader test code.

Write ToF distance sensor test code.

Write servo gate test code.

Write button/LED test code.

Record sensor readings, test results, and failure notes.

Create only a basic state machine draft in Week 1; full integration comes later.

Deliverables: test code for each module, testing checklist, sensor calibration notes, initial state machine draft, test log.

# 6. Collaboration Interfaces

| Interface | How to avoid conflict |
| --- | --- |
| Person 1 + Person 2 | Person 2 designs mechanisms; Person 1 integrates them into the main chassis CAD and final mounting layout. |
| Person 2 + Person 3 | Person 2 provides servo/button/LED mechanical locations; Person 3 checks cable reach, connector access, and power routing. |
| Person 3 + Person 4 | Person 3 wires hardware; Person 4 runs code and logs data. Tests must be done together. |
| Person 1 + Person 3 | Person 3 provides board sizes, connector access needs, battery dimensions, and cable routing requirements to Person 1. |

# 7. 3-Day Week 1 Sprint Plan

Because trial runs begin in Week 3, Week 1 must produce a manufacturable prototype plan and start real module testing. The goal is not perfection; the goal is to expose problems early.

## Day 1: Finalise concept + start CAD + confirm Arduino upload

| Area | Tasks | Minimum result |
| --- | --- | --- |
| All team | Agree on final layout: two-wheel differential drive, two-layer acrylic chassis, angled hopper, front bumper, front ToF, bottom RFID. | Concept fixed; no major architecture changes after this unless a test fails badly. |
| CAD | Start chassis layout and component footprint placement. | Main dimensions and component blocks placed. |
| Mechanism | Sketch hopper, servo gate, single-seed pocket, and revive bumper. | Mechanism direction confirmed. |
| Electronics | Connect Arduino GIGA and test upload. | Arduino upload works. |
| Software | Prepare motor/IR/RFID/servo/button test code templates. | Test code framework ready. |

## Day 2: First CAD version + priority module tests

| Area | Tasks | Minimum result |
| --- | --- | --- |
| CAD | Add mounting points, check size, integrate mechanism interfaces. | First chassis CAD mostly complete. |
| Mechanism | Complete first hopper/servo/bumper CAD and provide hole/interface dimensions. | Mechanism CAD first version ready. |
| Electronics + Software | Priority tests: Motoron + one motor, reflectance sensor black/white readings, servo movement. If time: RFID, ToF, button/LED. | At least 2-3 modules tested and logged. |

Day 2 priority note: Do not over-promise every module. Must test at least motor or IR sensor plus one other key module. RFID, ToF, and button/LED are important, but can continue into Day 3 if wiring or library issues occur.

## Day 3: Export prototype files + record test results + Week 2 fix list

| Area | Tasks | Minimum result |
| --- | --- | --- |
| CAD | Export DXF/STL for first prototype manufacturing. Check dimensions and component clearances. | Prototype manufacturing files ready. |
| Mechanism | Finalise printable hopper/servo/bumper parts; check if hopper fits 5 x 12.7 mm seeds. | Mechanism files ready for prototype. |
| Electronics | Finish pin assignment, wiring diagram, power plan, and cable management plan. | Pin table and wiring plan ready. |
| Software | Record all test results, sensor readings, failures, and create basic state machine outline. | Test log and Week 2 software plan ready. |

# 8. Week 2 and Week 3 Roadmap

| Week | Main goal | Minimum target |
| --- | --- | --- |
| Week 2 | Manufacture, assemble, and start integration. | First prototype assembled; motors can move; IR line following starts; RFID reader mounted/tested; servo seed gate tested on the robot; revive button + LED works. |
| Week 3 before trial run | Basic task flow, even if imperfect. | Robot can move along a line, read RFID, trigger seed release using a real or simulated server response, stop for obstacles, respond to kill/emergency logic, and produce a clear bug list after trial. |

# 9. Main Risks and Controls

| Risk | Control / mitigation |
| --- | --- |
| Reflectance sensor unreliable due to height or lighting | Use adjustable-height bracket; test black/white values early. |
| RFID reader cannot read reliably through chassis or at chosen height | Mount under chassis near ground; test read distance before finalising bottom plate. |
| Seed gate releases more than one seed | Use a single-seed pocket and test repeated releases. |
| CAD becomes too crowded | Keep target size below limit but leave service space; use two-layer layout. |
| Electronics/software testing blocked by wiring problems | Person 3 and Person 4 test together; maintain pin table. |
| Final wiring not allowed because of breadboard use | Use breadboard/jumpers only for Week 1 tests; plan perfboard/soldered board for final. |
| Week 3 trial run arrives before integration is ready | Focus Week 1 on manufacturable prototype and module tests; focus Week 2 on integration, not extra features. |

# 10. End-of-Week-1 Minimum Success Criteria

Final concept agreed.

First chassis CAD layout completed and exportable for prototype manufacturing.

Hopper and revive mechanism first version designed.

Arduino upload confirmed.

Motoron + at least one motor test started or completed.

Reflectance sensor black/white reading test started or completed.

Servo movement test completed.

RFID reader test started if time and wiring allow.

Pin assignment table completed.

Wiring diagram first version completed.

Week 2 manufacturing/debugging fix list written.